NTP & Router Clock

Computer Networks (CCNA) — Short Revision Notes

# 1. Why Time Matters

## What?

Every device has an internal clock. View it with show clock. Default zone is UTC.

⭐ Important: Accurate time is mainly needed for accurate logs (Syslog) for troubleshooting.

show clock / show clock detail  |  show logging (wrong clocks = confusing logs)

- * before the time = time is not authoritative.

- show clock detail shows the time source: hardware calendar (default), user configuration, or NTP.

- Hardware clock drifts, so it is not an ideal source.

Manual time: clock set (with ? help)  |  calendar set

# 2. Clock vs Calendar

| Clock | Calendar |
| --- | --- |
| Software clock, used by IOS | Hardware clock, survives reboot / power loss |
| clock set ¦ show clock | calendar set ¦ show calendar |
| Lost on reboot | Used to initialize the clock at boot |

R1# show clock detail

R1# clock set 14:30:00 27 Dec 2020        ! software clock

R1# calendar set 14:35:00 27 Dec 2020     ! hardware clock

R1# clock update-calendar                 ! clock    -> calendar

R1# clock read-calendar                   ! calendar -> clock

Syncing clock and calendar: update-calendar and read-calendar

Hardware clock keeps time after restart and initializes the software clock

- They are separate; usually you want them synchronized.

- After a reboot the clock may show a default date (e.g. 1993). Fix: clock read-calendar.

# 3. Time Zone & DST

R1(config)# clock timezone JST 9

R1(config)# clock summer-time EDT recurring 2 Sunday March 02:00 1 Sunday November 02:00

- clock timezone name hours-offset [minutes-offset] (hours: -23 to 23).

- clock summer-time name recurring start end [offset] (offset: 1-1440 min).

- First week day month time = start of DST; second = end of DST.

- After setting a timezone, clock set takes local time.

clock timezone  |  clock summer-time recurring

Start / End of DST  |  Command summary

# 4. NTP (Network Time Protocol)

## What / Why?

NTP syncs time automatically over a network. Manual time is not scalable and drifts.

⭐ Important: NTP uses UDP port 123.

- Clients request time from servers. A device can be client and server at the same time.

- Accuracy: about 1 ms on a LAN, about 50 ms over the Internet.

- Stratum = distance from the reference clock.

- NTP works only in UTC, so set the timezone on each device.

NTP basics

Example: PC syncing to time.windows.com  |  nslookup shows the NTP server IPs

## Stratum Hierarchy

Reference clock (atomic / GPS)  = Stratum 0

|

Stratum 1  (primary servers)    <-> peers

|

Stratum 2  (secondary servers: server + client mode)

|

Stratum 3 ...                   max usable = 15

- Lower stratum = preferred.

- Devices at the same stratum can peer to improve accuracy.

- A client can sync to multiple servers.

Reference clocks = stratum 0

NTP hierarchy (symmetric active mode = peering)

Primary vs secondary servers

## Cisco NTP Modes

| Mode | Command |
| --- | --- |
| Client | ntp server <ip> |
| Server | ntp master [stratum] (or a synced client acts as a server) |
| Symmetric active (peer) | ntp peer <ip> |

# 5. NTP Configuration

Lab topology: R1 syncs to Google servers

R1(config)# ntp server 216.239.35.0 prefer   ! prefer = favoured server

R1(config)# ntp server 216.239.35.4

R1(config)# ntp peer 10.0.23.2               ! same-stratum peer

R1(config)# ntp update-calendar              ! NTP time -> hardware calendar

R1(config)# ntp master [stratum]             ! act as source (default 8)

R1(config)# ntp source loopback0             ! source interface for NTP

R1# show ntp associations

R1# show ntp status

show ntp associations

show ntp status (stratum 2, reference = Google server)

- R1 syncs to Google (stratum 1), so R1 becomes stratum 2 and can serve others.

- R2 syncs to R1, so R2 becomes stratum 3.

- show ntp status shows: synchronized or not, stratum, reference IP.

## show ntp associations symbols

| Symbol | Meaning |
| --- | --- |
| * | sys.peer (chosen server) |
| # | selected |
| + | candidate |
| - | outlier |
| x | falseticker |
| ~ | configured |

## UTC & update-calendar

⭐ Important: NTP uses only UTC. Configure the right timezone on each device.

`ntp update-calendar` updates the hardware clock with NTP time

## Loopback as NTP source

- ntp source loopback0 uses the loopback IP as the source, so it is stable even if a physical link fails.

R1 loopback 10.1.1.1 as NTP source; R2 becomes stratum 3

- When a client has 2 servers, the lower stratum is preferred.

R3 with two servers: stratum 2 chosen over stratum 3

## NTP server mode (ntp master)

- ntp master makes the router a time source when it has no real one.

- Default stratum = 8. Range 1-15.

`ntp` options list  |  `ntp master` gives stratum 8

# 6. NTP Authentication (Optional)

## Why?

Clients sync only to intended (trusted) servers.

ntp authenticate                        ! enable

ntp authentication-key 1 md5 <key>      ! create key

ntp trusted-key 1                       ! trust the key

ntp server <ip> key 1                   ! use key for this server

ntp peer   <ip> key 1                   ! for peers

- The server (R1) needs only the first 3 commands. Clients also need ntp server ... key.

Authentication commands

Authentication config on R1, R2, R3 (see correction in Doubts)

## Full command summary

# 🔥 Quick Revision

- Accurate time = accurate logs = easier troubleshooting.

- Clock = software, calendar = hardware. Sync with clock update-calendar / clock read-calendar.

- NTP: UDP 123, works in UTC, set timezone per device.

- Stratum 0 = reference clock, 1 = primary, max 15. Lower is preferred.

- ntp server = client, ntp peer = symmetric active, ntp master = server (default 8).

- * in show ntp associations = chosen peer.

- Authentication uses MD5 keys and is optional.

# 📝 Revision Comments & Doubts

No handwritten doubts were in the slides. These are the points to fix or watch.

## Correction: typo in R3 authentication slide

Answer: The R3 block shows R2(config)#ntp peer 10.0.23.1 key 1. It should be R3(config)#ntp peer 10.0.23.1 key 1.

Remember: Each router peers with the OTHER router's IP.

## Doubt: Stratum 15 vs 16

Answer: 15 is the highest usable stratum. Stratum 16 means unsynchronized.

Remember: Max usable = 15.

## Doubt: Why use ntp update-calendar?

Answer: NTP sets only the software clock. Without this command the calendar stays wrong and re-initializes the clock incorrectly after a reboot.

Remember: NTP updates clock; update-calendar also updates hardware.

## Note: MD5 is weak

Answer: MD5 is what CCNA covers, but it is considered weak in real networks. Authentication protects who you sync from, not the time data.

Remember: Authentication = trusted source, not encryption.

# 🎯 Exam Points

| Item | Value |
| --- | --- |
| NTP port | UDP 123 |
| Default timezone | UTC |
| Default time source | Hardware calendar (* = not authoritative) |
| ntp master default stratum | 8 |
| Max stratum | 15 |
| Accuracy | ~1 ms (LAN), ~50 ms (Internet) |
| Clock to calendar | clock update-calendar |
| Calendar to clock | clock read-calendar |
| Peer mode name | Symmetric active |

## Common questions

- Which command syncs the calendar to the clock? clock update-calendar

- What is stratum? Distance from the reference clock.

- Which NTP server is preferred? The lowest stratum.

- Which command is NOT needed on the NTP server for authentication? ntp server ... key

- Why does NTP need a timezone configured? NTP only carries UTC.

## All commands

clock set | calendar set | clock timezone | clock summer-time recurring

clock update-calendar | clock read-calendar

ntp server | ntp peer | ntp master | ntp source | ntp update-calendar

ntp authenticate | ntp authentication-key | ntp trusted-key

show clock [detail] | show calendar | show ntp associations | show ntp status